🔌 Pin Connections (ESP32)

🟢 Gas Sensors

| Sensor | Pin | Type | Notes |
| --- | --- | --- | --- |
| CJMCU-4541 NOx output | GPIO34 | Analog input | (ADC1, input only) |
| CJMCU-4541 CO output | GPIO35 | Analog input | (ADC1, input only) |
| CJMCU-4541 Preheat | GPIO25 | Digital output | Set HIGH to enable heater |
| MQ135 Air Quality | GPIO32 | Analog input | (ADC1 channel) |

🌡️ Environmental Sensor

| Sensor | Pin | Type | Notes |
| --- | --- | --- | --- |
| DHT22 | GPIO4 | Digital input | Needs 4.7kΩ pull-up resistor between data and 3.3V |

🔥 Flame Sensor

| Sensor | Pin | Type | Notes |
| --- | --- | --- | --- |
| Flame Detector | GPIO27 | Digital input | Many modules output LOW when flame is detected |

📡 Ultrasonic Sensors (HC-SR04 ×4)

| Sensor | TRIG Pin | ECHO Pin | Notes |
| --- | --- | --- | --- |
| Ultrasonic #1 (Front) | GPIO12 | GPIO14 |  |
| Ultrasonic #2 (Left) | GPIO26 | GPIO33 |  |
| Ultrasonic #3 (Right) | GPIO18 | GPIO19 |  |
| Ultrasonic #4 (Back) | GPIO23 | GPIO5 |  |

⚠️ Important Notes for Ultrasonics

- ECHO pins output 5V pulses → protect ESP32 (3.3V max). Use a voltage divider (2 resistors, e.g., 1k + 2k) on each ECHO line.

- TRIG pins are fine (ESP32 outputs 3.3V, HC-SR04 accepts it).

⚡ Powering

- ESP32: 5V to VIN, GND to GND.

- Sensors:

  - CJMCU-4541 → 5V (heater), GND. Outputs are safe analog voltages.

  - MQ135 → 5V VCC, GND, output → GPIO32.

  - DHT22 → 3.3V VCC (preferred), GND, data → GPIO4.

  - Flame → 3.3V VCC, GND, digital out → GPIO27.

  - Ultrasonics → 5V VCC, GND shared, TRIG/ECHO as above.